# LAB Journal Serie 1

Thomas Baumann & Tobias Weissert

## Exercise 1

- Set up Git repo

- Set up LAB-Journal

- Group assingment nr: n113

- Familiarize with the virtual lab setup

- Search RHEL 7 Networking guide

- Router VM edit config of ENS4

Router:
Update /etc/resolve.conf
search n113.nslab.ch nslab.ch
search netlab.nslab.ch
nameserver 193.5.80.80

Router:

Update /etc/hostname
router.n113.nslab.ch

Router:

Update file: /etc/sysconfig/network-scripts/ifcfg-ens4
DEVICE=ens4
NM_CONTROLLED=no
TYPE=Ethernet
ONBOOT=yes
BOOTPROTO=none
IPADDR=193.5.80.113
PREFIX=27
GATEWAY=193.5.80.1
IPV4_FAILURE_FATAL=yes
Name="System eth0"

Router:

Update file: /etc/sysconfig/network-scripts/ifcfg-ens3
DEVICE=ens3
NM_CONTROLLED=no
TYPE=Ethernet
ONBOOT=yes
BOOTPROTO=none
IPADDR=193.5.82.129
PREFIX=27
GATEWAY=193.5.82.1
IPV4_FAILURE_FATAL=yes
Name="System eth0"

Router:

Update /etc/sysctl.conf
Net.ipv4.ip_forward = 1

sysctl -p /etc/sysctl.conf
systemctl restart network

Router:

ping 8.8.8.8 ✓
traceroute 8.8.8.8 ✓
ping google.com ✓

Client: - Set IP to manual: 193.5.82.128/27 Gateway: 193.5.82.129 - Set DNS Server to 193.5.80.80

### 5. März 2019

Tobias Weissert & Thomas Baumann Network capture

ARP capture

## Exercise 2

Router: ping 193.5.82.100 [Redirect host, nexthop: 193.5.80.112]

Router: ip route add 193.5.82.96/27 via 193.5.80.112 dev ens4

Make route persistent create file /etc/sysconfig/network-scripts/route-ens4 193.5.82.96/27 via 193.5.80.112 dev ens4

## Exercise 3

Router: change /etc/sysconfig/network-scripts/ifcfg-ens3 and ifcfg-ens4

ONBOOT=no

Router: add to /etc/quagga/zebra.conf

log file /var/log/quagga/zebra.log

systemcpl start zebra

vtysh:
conf t
interface ens3
ip address 193.5.82.129/27
interface ens4
ip address 193.80.113/27
ip route 193.5.82.96/27 193.5.80.112
ip route 193.5.82.96/27 ens4
ip route 0.0.0.0/0 193.5.80.1
write mem

### 12. März 2019

Tobias Weissert & Thomas Baumann

vtysh:
conf t
no ip route 193.5.82.96/27 193.5.80.112
no ip route 193.5.82.96/27 ens4
no ip route 0.0.0.0/0 193.5.80.1
no ip address 193.5.82.129/27

ip address 193.5.82.129/24
ping 8.8.8.8 ✓

## Exercise 4

Router: add to /etc/quagga/ripd.conf

log file /etc/quagga/ripd.conf

systemctl start ripd
Log contains: RIPd starting

chown quagga.quagga /var/log/qzagga/ripd.conf
vtysh
no ip route 0.0.0.0/ 193.5.80.1
conf t key chain demonet
key 1
key-string demo$rip
interface ens4
ip rip authentication mode md5
ip rip authentication key-chain demonet

router rip
redistribute connected
network 193.5.80.0/24
network ens4
distance 100 193.5.80.0/24
ping 8.8.8.8 ✓

## Exercise 5

Router: add to /etc/quagga/ospfd.conf

log file /var/log/quagga/ospfd.conf

systemctl start ospfd
ospf starting
chown quagga.quagga /var/log/qzagga/ospfd.conf

vtysh coinf t
router ospf
ospf router-id 193.5.80.113
interface ens4
ip ospf authentication message-digest
ip ospf message-digest-key 1 md5 demo$ospf
redistribute connected
network 193.5.80.0/24 area 0.0.0.0
area 0.0.0.0 range 193.5.80.0/24
area 0.0.0.0 authentication message-digest
systemctl enable ospfd

OSPF Capture

## Exercise 6

### 19. März 2019

Tobias Weissert & Thomas Baumann

RIP has a higher priority

router rip
distance 120 193.5.82.160/27
ip route 192.5.80.1 0.0.0.0/0 130

Static route

### 26. März 2019 IETF 104

# LAB Journal Serie 2

### 2. April 2019

Tobias Weissert & Thomas Baumann

ip -6 addr

ping6 www.switch.ch ✓

Router advertisment

Source address in the router advertisment is the virtual network adapter of the VM host.

vtysh conf t interface ens4
ipv6 address 2001:620:500:FF00::FF0D/64
ipv6 address FE80::FF0D/64

vtysh conf t interface ens3
ipv6 address 2001:620:500:FF0D::1/64
ipv6 address FE80::1/64
write mem

vtysh conf t interface ens4
ipv6 route ::/0 FE80::FC54:FF:FEE7:8557 250
write mem

ping6 switch.ch ✓
ping6 -i ens4 fe80::1 ✓

### 09. April 2019

Tobias Weissert & Thomas Baumann ## Exercise 9 we prefer quagga

vtysh conf t interface ens3
no ipv6 ns suppress-ra
ipv6 nd prefix 2001:620:500:FF0D::/64
write mem

edit /etc/sysctl.conf

net.ipv6.conf.all.forwaring = 1

client

ip a
ipv6: 2001:620:500:FF0D:1116:6EE0:E63F:5D24/64 ✓
ping6 2001:620:FF00::FF0D ✓

ntptime
ifconfig ens3
echo e0576a5c5d45a0005054fffeaa354b | sha1sum - | cut -c31-40
vtysh interface ens3
ipv6 address fdf8:f06a:90f5::/48
ipv6 nd prefix fdf8:f06a:90f5::/48

## Exercise 10

edit /etc/quagga/ripngd.conf

log file /var/log/quagga/ospf6.conf

chown quagga.quagga /var/log/quagga/ripngd.conf
vtysh
router ripng
redistribute connected

### 23. April 2019

Tobias Weissert & Thomas Baumann

# Serie 3 DHCP and DNS

## Exercise 12

edit /etc/sysconfig/network

NETWORKING=yes
NETWORKING_IPV6=yes
NOZEROCONF=yes
GATEWAY=193.5.82.129
IPV6_DEFAULTDEV=ens3
IPV6_DEFAULTGW=FE80::1

edit /etc/sysconfig/network-scripts/ifcfg-ens3

BOOTPROTO=static
DEVICE=ens3
ONBOOT=yes
PREFIX=27
IPADDR=193.5.82.130
IPV6INIT=yes
IPV6_AUTOCONF=no
IPV6ADDR=2001:620:500:FF0D::20/64
NM_CONTROLLED=no

hostnamectl set-hostname ns.n113.nslab.ch
rpm -qa | grep dhcp

edit /etc/dhcp/dhcpd.conf

option domain-name "ns113.nslab.ch";
option domain-name-servers 193.5.82.130, 193.5.80.80;

default-lease-time 300;
max-lease-time 7200;

log-facility local7;

subnet 193.5.82.128 netmask 255.255.255.224 {
  range 193.5.82.144 193.5.82.158;
  option routers 193.5.80.113;
}

systemctl start dhcpd
systemctl enable dhcpd

Change Client 1 from fix IP address to DHCP Client 1 got the first IP address in the range 193.5.82.144

edit /etc/dhcp/dhcpd.conf

host client1 {
  hardware ethernet 52:54:00:35:84:52;
  fixed-address 193.5.82.150
}

Client 1 got the new IP address 193.5.82.150

## Exercise 13

edit /etc/dhcp/dhcpd6.conf option dhcp6.name-servers 2001:620:500:ff0d::20; option dhcp6.domain-search “n113.nslab.ch”;

subnet6 2001:620:500:ff0d::/64 {
  range6 2001:620:500:ff0d::40 2001:620:500:ff0d::2000;
}

dhcp6 start

client1 got a ipv6 address

vtysh conf interface ens3
ipv6 nd managed-config-flag
ipv6 nd other-config-flag
ipv6 nd ra-invervall 60
no ipv6 nd suppress-ra
write mem

host client1 {
  hardware ethernet 52:54:00:35:84:52;
  fixed-address6 2001:620:500:ff0d::50;
}

dhclient -6 -r
dhclient -6

DHCP 6 Capture

## Exercise 14

Add /var/named/named.conf

zone "." IN{
  type hint;
  file "/var/named/named.cache";
};

zone "n113.nslab.ch" {
  type master;
  file "/var/named/fwd-n113.nslab.ch";
};

update fwd-n113.nslab.ch

;
; BIND Zone File
;
$TTL    300
@       IN      SOA     ns.n113.nslab.ch root.n113.nslab.ch (
                        2018050301      ; Serial
                        600             ; Refresh
                        300             ; Retry
                        7200            ; Expire
                        1200 )          ; Negative Cache TTL

@       IN      NS      ns
ns      IN      A       193.5.82.130
ns      IN      AAAA    2001:620:500:ff0D::20

systemctl named start

less var/log/messages > all zones loaded and running

add to named.conf

listen-on port 53 {any}
listen-on-v6 port 53 {any}

client01

dig any ns.n113.nslab.ch

## Exercise 15

create file /var/named/rev-n113.nslab.ch

;
; BIND Zone File
;
$TTL    300
@       IN      SOA     ns.n113.nslab.ch root.n113.nslab.ch (
                        2018050301      ; Serial
                        600             ; Refresh
                        300             ; Retry
                        7200            ; Expire
                        1200 )          ; Negative Cache TTL

          IN     NS     ns.113.nslab.ch.
130       IN     PTR    ns.113.nslab.ch.

create file /var/named/rev6-n113.nslab.ch

;
; BIND Zone File
;
$TTL    300
@       IN      SOA     ns.n113.nslab.ch root.n113.nslab.ch (
                        2018050301      ; Serial
                        600             ; Refresh
                        300             ; Retry
                        7200            ; Expire
                        1200 )          ; Negative Cache TTL

;D.0        IN     NS     ns.113.nslab.ch.
;D.0        IN     PTR    ns.113.nslab.ch.
@           IN     NS     ns.113.nslab.ch.
0.2.0.0.0.0.0.0.0.0.0.0.0.0.0 IN   PTR   ns.n113.nslab.ch.

client01

dig any 193.5.82.130

## Exercise 16

already done earlier

## Exercise 17

nslookup sbb.ch

nslookup for sbb.ch

named.conf

include "/etc/rndc.key";

controls {
        inet 127.0.0.1 allow { localhost; } keys { "rndc-key"; };
};

systemctl restart named
rndc status
rndc dumpdb -cache

cat /var/named/data/cache_dump.db

## Exercise 18

add to /etc/dhcpd.conf

update-optimization false;
update-static-leases false;

create rndc.conf

server localhost {
  key  "rndc-key";
};
key "rndc-key" {
  algorithm hmac-md5;
  secret "<key>";
};

rndc dumpdb -cache

update dhcpd.conf

# update dns config each time
update-optimization false;
update-static-leases true;

key DHCP_UPDATER {
 algorithm hmac-md5;
 secret Qq6gGm8yExOc7ltYRutSV47prHBMiG2Ty9okFt1zEvLmwfBGZ8UEO3VyG5uq;
};

zone n113.nslab.ch. {
  primary 193.5.82.130;
  key DHCP_UPDATER;
}

zone 128.82.5.193.in-addr.arpa. {
  primary 193.5.82.130;
  key DHCP_UPDATER;
}

add ipv6

zone D.0.F.F.0.0.5.0.0.2.6.0.1.0.0.2.ip6.arpa. {
  primary ns.113.nslab.ch;
  key DHCP_UPDATER;
}

# Exercise 4

Edit etc/sysconfig/network-scripts/ifcfg-ens

BOOTPROTO=static
DEVICE=ens3
ONBOOT=yes
NM_CONTROLLED=no
IPADDR=193.5.82.131
NETMASK=255.255.255.224
GATEWAY=193.5.82.225
IPV6_DEFAULTDEV=ens3
IPV6_DEFAULTGW=FE80::1
IPV6ADDR=2001:620:500:FF0D::25
IPV6INIT=yes
IPV6_AUTOCONFIG=no
NETWORKING_IPV6=yes
NOZEROCONF=yes

hostnamectl set-hostname mail.n116.nslab.ch
systemctl restart network

Add DNS Server to Server2 Add to sysconfig/resolf.conf

nameserver localhost

Check internet connection ✅

Add DNS entry for Mail fwd-ns113.nslab.ch

‘DNS Configuration’

Edit main.cf

myhostname = mail.n113.nslab.ch
mydomain = n113.nslab.ch
inet_interfaces = all
mydestination = $myhostname, localhost.$mydomain, localhost, $mydomain, mail.n113.nslab.ch

telnet localhost 25

Trying 193.5.82.131...
Connected to mail.
Escape character is '^]'.
220 mail.n113.nslab.ch ESMTP Postfix
EHLO n113.nslab.ch
250-mail.n113.nslab.ch
250-PIPELINING
250-SIZE 10240000
250-VRFY
250-ETRN
250-ENHANCEDSTATUSCODES
250-8BITMIME
250 DSN
MAIL FROM: user@n113.nslab.ch
250 2.1.0 Ok
RCPT TO: user@n113.nslab.ch
250 2.1.5 Ok
DATA
354 End data with <CR><LF>.<CR><LF>
Subject: test
test test
.
250 2.0.0 Ok: queued as 6E4F723977E7
QUIT
221 2.0.0 Bye
Connection closed by foreign host.

‘Mail Configuration’

‘Mail Log’

‘Mail Inbox’

sudo apt install mailutils Install satelite system with n113.nslab.ch as relay

echo test | mail -s "das ist ein Test" thomas.baumann@students.bfh.ch

‘Send and recieve Mail’

dovecot already installed create file /etc/dovecot/local.conf

systemctl start dovecot
telnet mail.n113.nslab.ch 110

‘dovecot’

systemctl enable dovecot

edit /etc/dovecot/conf.d/10-ssl.conf

ssl = no
disable_plaintext_auth = no

‘Thunderbird’

‘IMAP’

install https, php mod_ssl

create file mail.conf in /etc/https/conf.d systemctl start httpd http://mail.n113.nslab.ch works

install certbot python2-certbot-apache

get lets encrypte certificate

https://mail.n113.nslab.ch works

‘https’

edit /etc/postfix/master.cf and enable

systemctl enable saslauthd systemctl restart postfix

- swaks -tlso -t user@n113.nslab.ch

[root@mail postfix]# swaks -tlso -t user@n113.nslab.ch
=== Trying mail.n113.nslab.ch:25...
=== Connected to mail.n113.nslab.ch.
<-  220 mail.n113.nslab.ch ESMTP Postfix
 -> EHLO mail.n113.nslab.ch
<-  250-mail.n113.nslab.ch
<-  250-PIPELINING
<-  250-SIZE 10240000
<-  250-VRFY
<-  250-ETRN
<-  250-STARTTLS
<-  250-ENHANCEDSTATUSCODES
<-  250-8BITMIME
<-  250 DSN
 -> STARTTLS
<-  220 2.0.0 Ready to start TLS
=== TLS started with cipher TLSv1.2:ECDHE-RSA-AES256-GCM-SHA384:256
=== TLS no local certificate set
=== TLS peer DN="/CN=mail.n113.nslab.ch"
 ~> EHLO mail.n113.nslab.ch
<~  250-mail.n113.nslab.ch
<~  250-PIPELINING
<~  250-SIZE 10240000
<~  250-VRFY
<~  250-ETRN
<~  250-ENHANCEDSTATUSCODES
<~  250-8BITMIME
<~  250 DSN
 ~> MAIL FROM:<user@mail.n113.nslab.ch>
<~  250 2.1.0 Ok
 ~> RCPT TO:<user@n113.nslab.ch>
<~  250 2.1.5 Ok
 ~> DATA
<~  354 End data with <CR><LF>.<CR><LF>
 ~> Date: Fri, 14 Jun 2019 13:01:28 +0200
 ~> To: user@n113.nslab.ch
 ~> From: user@mail.n113.nslab.ch
 ~> Subject: test Fri, 14 Jun 2019 13:01:28 +0200
 ~> Message-Id: <20190614130128.005228@mail.n113.nslab.ch>
 ~> X-Mailer: swaks v20170101.0 jetmore.org/john/code/swaks/
 ~>
 ~> This is a test mailing
 ~>
 ~> .
<~  250 2.0.0 Ok: queued as 6F5DC32A9DE7
 ~> QUIT
<~  221 2.0.0 Bye
=== Connection closed with remote host.

- Edit /etc/dovecot/local.conf

- systemctl restart dovecot

- Test TLS

- To limit access to “dovecot” to POP3S/IMAP4

- Edit /etc/dovecot/local.conf and check log

Jun 14 13:49:36 mail dovecot: master: Dovecot v2.2.36 (1f10bfa63) starting up for imap, pop3 (core dumps disabled)

- Wireshark TLS

- Receive Mail works

- Config MUA

- Sending and receving Mails works! # 25

yum install --enablerepo=epel roundcubemail

- Edit /etc/httpd/conf.d/roundcubemail.conf and verify if it works

- Generate Roundcube conf and check if everything is ok:

- Login to: https://mail.n124.nslab.ch/roundcubemail/

- Test Send and receive Mails

- Ingoing

- Outgoing